11:30  – 11:50 X-Ray methods for  Anti-counterfeiting analysis of Electronics

Speaker: Marc Jousse (CTO @ Data Peanuts, Consultant Expert @ Retronix Europe)

Chair: Laurent Dupuy

Location: IBS Seminar Hall

Abstract: Counterfeit Detection in Electronics

# “Laboratory Analysis and Digital Traceability

Early detection and prevention of counterfeit electronic components are essential to protect product reliability and safety across the supply chain.

Modern laboratories follow recognized standards such as AS6081, AS6171, AS5553, CCAP-101, and IDEA-1010, combining visual, analytical, and electrical tests to determine authenticity.

# Laboratory Workflow

## 1. Documentation & Packaging Inspection

Verification of purchase records, lot codes, labeling, and packaging integrity. Discrepancies or missing certificates often indicate high risk.

## 2. External Visual Inspection (EVI)

Microscopic examination for sanding, black-topping, re-marking, or altered fonts. Comparison against “known good parts” ensures consistency in logo, mold marks, and pin finish.

## 3. X-ray Fluorescence (XRF)

Non-destructive elemental analysis of plating and lead finish. Detects alloy inconsistencies or substitution of prohibited materials.

## 4. X-ray Imaging

Internal structural inspection to reveal counterfeit die, multiple dies, broken bond wires, or reworked assemblies not matching genuine parts.

## 5. Heated Solvent Test

Chemical test to expose resurfacing or counterfeit coatings that dissolve or deform under heat or solvent.

## 6. Decapsulation and Die Analysis

Destructive opening of the package to examine the silicon die markings, layout, and metallization under microscope—providing the most reliable authenticity evidence.

## 7. Electrical and Parametric Tests

Functional or curve-trace testing to compare electrical signatures with the manufacturer’s datasheet.

Each analysis results in a forensic report including photographs, X-ray images, and conclusions (Authentic / Suspect / Confirmed Counterfeit).

# From Counterfeit Detection to Digital Traceability

While laboratory analysis confirms authenticity at one point in time, the upcoming Digital Product Passport (DPP) will ensure continuous traceability across the component’s lifecycle.

Through the DPP, each component or Printed Board Assembly (PBA) can store and share:

- Manufacturer and batch identification

- Compliance certificates (RoHS, REACH, CE)

- Counterfeit analysis reports from accredited labs

- Repair, refurbishment, and recycling history

Data Peanuts, in collaboration with Retronix Europe, is developing a DPP framework tailored to electronics, turning every tested component into a traceable, verified, and recoverable asset.

By 2027, the DPP will become a central tool to ensure transparency, authenticity, and circularity across the European electronics market.

Counterfeit-free. Traceable. Sustainable. Go Circular.”